Micromouse Flood-Fill Robot

Complete Discussion Preparation Guide

ESP32  |  MPU6050  |  3x VL53L0X LiDAR  |  Quadrature Encoders  |  Web Dashboard

# 1. Project Overview

This is an autonomous Micromouse-style maze-solving robot. It navigates an 8x8 grid maze and finds its way to the center four cells (rows 3-4, columns 3-4) using the Flood Fill algorithm. The robot is controlled by an ESP32 microcontroller and uses three types of sensors for precise navigation: a gyroscope (IMU), LiDAR distance sensors, and wheel encoders.

The robot also hosts a live Wi-Fi web dashboard that lets you monitor sensor data, visualize the maze in real time, tune PD parameters, and send manual commands — all from a browser.

# 2. Hardware Components & Wiring

## 2.1 ESP32 Microcontroller

The ESP32 is the brain of the robot. It is a dual-core 32-bit microcontroller with built-in Wi-Fi and Bluetooth. It runs all the code: reading sensors, computing corrections, driving motors, hosting the web server, and executing the maze algorithm.

The ESP32 supports two hardware I2C buses (Wire and Wire1), which is critical here because we have 4 I2C devices and needed to separate them into two buses to avoid conflicts.

## 2.2 MPU6050 — Gyroscope / IMU

| Purpose | Measures angular velocity (gyro) and linear acceleration (accelerometer). Used primarily to track the robot's yaw (rotation angle). |
| --- | --- |
| Protocol | I2C — connected on the FIRST I2C bus (Wire) |
| SDA Pin | GPIO 19 (GYRO_SDA) |
| SCL Pin | GPIO 18 (GYRO_SCL) |
| I2C Address | 0x68 |
| Gyro Range | ±500 degrees/second |
| Filter | 21 Hz low-pass filter to reduce noise |

How it is used in code: Every loop iteration, the code reads gyroscope Z-axis data (g.gyro.z), converts it from radians/s to degrees/s by multiplying by 57.2958, then integrates over time (dt) to accumulate the yaw angle (currentYaw += gyroZ_raw * dt). A deadband of 1.5 degrees/s filters out gyro drift when the robot is nearly still.

## 2.3 VL53L0X — Time-of-Flight LiDAR Sensors (x3)

Three VL53L0X sensors are used: one facing left, one facing front, and one facing right. The VL53L0X is a laser time-of-flight sensor that measures distance by emitting a laser pulse and timing how long it takes to bounce back. It gives readings in millimeters.

| Protocol | I2C — all three on the SECOND I2C bus (Wire1) |
| --- | --- |
| SDA Pin | GPIO 5 (LIDAR_SDA) |
| SCL Pin | GPIO 17 (LIDAR_SCL) |
| Default I2C Address | 0x29 (all three have the same default!) |
| Left LiDAR XSHUT | GPIO 15 — reassigned to address 0x30 |
| Front LiDAR XSHUT | GPIO 4 — reassigned to address 0x31 |
| Right LiDAR XSHUT | GPIO 16 — kept at 0x29 |
| Range used | 0 to ~300mm inside the maze |

Why XSHUT pins? All three VL53L0X sensors have the same I2C address (0x29) by default. They can't all be on the same bus at the same address. The XSHUT pin lets you power-cycle each sensor individually and reprogram its address. In setup(), the code first pulls all XSHUT pins LOW (disabling all sensors), then enables them one by one and assigns each a unique address before enabling the next.

How addresses are assigned (from setup):

digitalWrite(XSHUT_LEFT, HIGH); delay(10);

lidarLeft.begin(0x30, false, &Wire1);   // Left assigned 0x30

digitalWrite(XSHUT_FRONT, HIGH); delay(10);

lidarFront.begin(0x31, false, &Wire1);  // Front assigned 0x31

digitalWrite(XSHUT_RIGHT, HIGH); delay(10);

lidarRight.begin(0x29, false, &Wire1);  // Right stays 0x29

All sensors are then set to continuous ranging mode with startRangeContinuous(), so they are always measuring in the background.

## 2.4 DC Motors + Motor Driver

Two DC motors drive the left and right wheels. A motor driver IC (compatible with L298N-style logic) is used. The driver takes two control pins per motor channel (IN1/IN2 for left, IN3/IN4 for right) and uses PWM to control speed and direction.

| Left Motor IN1 | GPIO 13 — forward PWM |
| --- | --- |
| Left Motor IN2 | GPIO 27 — backward PWM |
| Right Motor IN3 | GPIO 25 — forward PWM |
| Right Motor IN4 | GPIO 26 — backward PWM |
| Speed range | 0 to 255 (8-bit PWM via analogWrite) |
| Move Speed | 150 (default) |
| Turn Speed | 130 (default) |

How setMotors() works: It takes a left speed (L) and right speed (R), clamped to [-255, 255]. Positive value = forward (IN1=PWM, IN2=0). Negative value = backward (IN1=0, IN2=abs(value)). Zero = both pins LOW (coast). The same logic applies to the right motor via IN3/IN4.

Braking: stopMotors() applies a reverse pulse at brakeStrength (60) for brakeDurationMs (40ms) before setting both to zero. This active braking stops the robot faster and more precisely than just cutting power.

## 2.5 Quadrature Encoders (x2)

Each wheel has a quadrature encoder — a disk with slots that produces pulses as the wheel turns. Two signal channels (A and B) allow detection of both speed and direction.

| Left Encoder A | GPIO 33 (ENC_LEFT_A) — interrupt-driven |
| --- | --- |
| Left Encoder B | GPIO 32 (ENC_LEFT_B) |
| Right Encoder A | GPIO 34 (ENC_RIGHT_A) — interrupt-driven |
| Right Encoder B | GPIO 35 (ENC_RIGHT_B) |
| PPR | 360 pulses per revolution |
| Wheel Diameter | 32mm → Circumference = π × 32 = ~100.5mm |

How direction is detected: In the ISR (Interrupt Service Routine), if A and B are the same logic level when A fires, it's counted as forward (+1 tick). If they're different, it's backward (-1 tick). This is standard quadrature decoding.

Distance calculation: distance_mm = (ticks / PPR) * wheelCircumference = (ticks / 360) * 100.5mm

The ISRs are tagged IRAM_ATTR to ensure they run from RAM, not flash — this is critical on the ESP32 for reliable interrupt handling.

# 3. Code Structure — Line by Line

## 3.1 Libraries

#include <Wire.h>          // I2C communication (both buses)

#include <WiFi.h>          // ESP32 WiFi

#include <WebServer.h>     // HTTP server for the dashboard

#include <Adafruit_VL53L0X.h>   // LiDAR driver

#include <Adafruit_MPU6050.h>   // Gyroscope driver

#include <Adafruit_Sensor.h>    // Unified sensor abstraction layer

## 3.2 Global Variables — Key Ones

| currentYaw | Accumulated rotation angle in degrees. Starts at 0, incremented each loop by gyroZ * dt. |
| --- | --- |
| headingLock | The yaw angle saved when straight-line driving begins. The robot tries to keep currentYaw equal to this. |
| distLeft/Front/Right | Latest LiDAR readings in mm. |
| leftEncoderTicks / rightEncoderTicks | Volatile (ISR-updated) tick counters. Volatile means the compiler won't cache them — reads always come from memory. |
| robotRow, robotCol, robotDir | Robot's current position in the 8x8 grid. Dir: 0=North, 1=East, 2=South, 3=West. |
| mazeWalls[8][8] | Bitmask for known walls. Each cell stores 4 bits: bit0=N, bit1=E, bit2=S, bit3=W. |
| mazeFlood[8][8] | Flood fill distance values. 0 at center, higher as you move away. |
| robotState | Current state of the finite state machine. Controls all robot behavior. |

## 3.3 setup() — Initialization Sequence

1. Serial.begin(115200) — starts debug serial output.

2. Encoder pins are configured as INPUT and interrupts are attached to pins ENC_LEFT_A and ENC_RIGHT_A, triggering on CHANGE (both rising and falling edges). This gives double resolution.

3. Motor pins are set as OUTPUT and motors are stopped.

4. All three XSHUT pins are pulled LOW to reset all LiDAR sensors.

5. Wire.begin(19, 18) starts the first I2C bus for the MPU6050.

6. Wire1.begin(5, 17) starts the second I2C bus for all three LiDARs.

7. mpu.begin(0x68, &Wire) initializes the gyroscope. Gyro range set to ±500 deg/s, filter to 21 Hz.

8. LiDAR sensors are enabled one-by-one via their XSHUT pins and given unique addresses.

9. All LiDARs are set to continuous ranging mode.

10. WiFi.begin() connects to the network. If connected, the IP address is logged.

11. initMaze() and floodFill() set up the initial maze state.

12. HTTP routes are registered (/,  /data, /command, /settings, /fusioncfg, /correctcfg, /generalcfg, /mazecfg).

13. server.begin() starts the web server.

## 3.4 loop() — Main Loop

Runs continuously. Each iteration does:

- server.handleClient() — processes any incoming HTTP requests from the dashboard.

- Reads LiDAR sensors if a new reading is ready (non-blocking, using isRangeComplete()).

- Reads MPU6050 gyro and accelerometer via mpu.getEvent().

- Computes dt (time since last loop in seconds).

- Converts gyro Z from rad/s to deg/s. Applies deadband, then integrates: currentYaw += gyroZ * dt.

- Runs the state machine switch statement to decide what motors should do.

# 4. Straight-Line Control — 3-Sensor Fusion

This is one of the most sophisticated parts of the code. When driving straight, three independent correction signals are computed and blended together to keep the robot on a straight, centered path.

## 4.1 Layer 1 — Gyroscope PD Controller

The gyro measures how much the robot has rotated away from its locked heading. A PD (Proportional-Derivative) controller computes a correction:

gyroErr = currentYaw - headingLock

gyroDeriv = gyroErr - lastGyroErr   // Rate of change of error

gyroCorr = -(gyroKp * gyroErr + gyroKd * gyroDeriv)

Kp=1.50 (how strongly to correct the error), Kd=0.40 (how strongly to resist rapid changes). The negative sign means: if currentYaw > headingLock (drifted right), correction is negative → slow down left, speed up right → turn back left. Clamped to ±25.

## 4.2 Layer 2 — Encoder PD Controller

Compares the number of ticks on each wheel since lockHeading() was called:

encErr = relLeft - relRight   // If left > right, robot is veering right

encCorr = encKp * encErr

Kp=0.15. Clamped to ±20. This catches cases where one motor is slightly slower than the other despite the same PWM value.

## 4.3 Layer 3 — LiDAR Wall PD Controller

Uses the side LiDAR sensors to keep the robot centered between the walls. The ideal wall distance is 50mm (cell width 200mm, robot width ~100mm → (200-100)/2 = 50mm each side).

If both walls are visible: wallErr = distRight - distLeft (simple centering).

If only the left wall is visible: wallErr = (idealWallDist - distLeft) * 2 (maintain 50mm from left wall).

If only the right wall is visible: wallErr = (distRight - idealWallDist) * 2.

Only triggers if |wallErr| > 8mm (deadband) to avoid jitter. Uses Kp=0.20 and Kd=0.45.

## 4.4 Fusion

The three corrections are combined using weighted average:

fused = (0.55 * gyroCorr) + (0.25 * encCorr) + (0.20 * wallCorr)

If no walls are in range, the LiDAR weight (0.20) is redistributed proportionally between gyro and encoder so the weights always sum to 1.0. Final correction is clamped to ±25.

Applied to motors: Left motor = speed - correction. Right motor = speed + correction. So a positive correction (robot drifted left) speeds up the right and slows the left.

# 5. Maze-Solving Algorithm — Flood Fill

## 5.1 What is Flood Fill?

Flood Fill is a BFS (Breadth-First Search) algorithm that assigns a "distance to goal" value to every cell in the maze. The four center cells start at 0. Their neighbors get 1, their neighbors' neighbors get 2, and so on — like water flooding from the center.

The robot always moves to whichever adjacent cell has the lowest flood value. This guarantees the shortest possible path to the center, even in a partially-explored maze.

## 5.2 Data Structures

| mazeWalls[8][8] | Each cell is a byte. Bits 0-3 represent walls: N=0x01, E=0x02, S=0x04, W=0x08. E.g., 0x05 means walls on North and South. |
| --- | --- |
| mazeFlood[8][8] | Stores the flood-fill distance for every cell. Initialized to 255 (unknown/unreachable). Center 4 cells = 0. |
| mazeVisited[8][8] | Boolean — true if the robot has physically been at that cell and scanned walls. |
| pathR[256], pathC[256] | Records the robot's path through the maze (row and column of each cell visited). |
| DR[], DC[] | Direction delta arrays: DR={-1,0,1,0}, DC={0,1,0,-1} for N,E,S,W. |

## 5.3 floodFill() Function

Uses BFS starting from the 4 center cells:

1. Reset all flood values to 255.

2. Set center cells (rows 3-4, cols 3-4) to 0 and add them to the queue.

3. For each cell dequeued, check all 4 directions. If no wall blocks the way and the neighbor's flood value is greater than current+1, update it and enqueue it.

4. Repeat until queue is empty.

This is re-run every time the robot discovers a new wall, because a new wall might change the shortest path.

## 5.4 scanMazeWalls() Function

Called each time the robot arrives at a new cell. Takes 5 LiDAR samples with 20ms delay between each. Uses majority vote (3 out of 5 must agree) before marking a wall, to avoid false positives from noisy sensor readings.

Walls are recorded using setWallBilateral(), which marks the wall on BOTH sides — e.g., if cell (3,2) has a wall to the East, then cell (3,3) automatically gets a wall to the West. This prevents the algorithm from trying to cross from the other side.

## 5.5 findBestDirection() Function

After floodFill(), this finds which neighbor has the lowest flood value. Tie-breaking order is: straight ahead > right > left > backward. This preference for going straight reduces unnecessary turns.

## 5.6 State Machine — Maze States

| MAZE_SCAN | Robot pauses for stable readings, then calls scanMazeWalls(). Transitions to MAZE_DECIDE. |
| --- | --- |
| MAZE_DECIDE | Checks if at target. If not, runs floodFill() + findBestDirection(). Decides whether to go straight, turn 90°, turn 180°, or reverse. |
| MAZE_TURN_BACKUP | Backs up mazeTurnBackupMM (60mm) to create clearance before pivoting. |
| MAZE_TURN_PIVOT | Spins in place until currentYaw reaches the target angle (85° for 90-turns, 170° for 180-turns). |
| MAZE_TURN_ADVANCE | Drives forward turnForwardMM (60mm) after the pivot. Updates robotDir. |
| MAZE_FORWARD | Drives forward mazeMoveDist mm using fused correction. Detects mid-move obstacles. |
| MAZE_REVERSE | Drives backward one cell on a known path (no need to turn around). Used for backtracking on already-visited cells. |
| MAZE_DONE | Target reached. Motors stop. Stays here until user resets. |

## 5.7 Turn Execution

A 90-degree turn follows a 3-phase sequence: Backup (move backward ~60mm to give the robot room to pivot) → Pivot (spin one wheel in place until gyro reaches 85°) → Advance (drive forward ~60mm to center on the next cell).

A 180-degree turn is similar but uses 170° as the gyro target. If the robot needs to go 180° toward a KNOWN cell, it reverses instead of turning (much more efficient). If the path is UNKNOWN (unvisited), it must face that direction to scan walls, so it physically turns around.

# 6. Web Dashboard (Wi-Fi Interface)

The ESP32 hosts an HTTP web server on port 80. When you connect to its IP address from a browser on the same network, you get a full dashboard.

## 6.1 Routes

| GET / | Returns the full HTML/CSS/JavaScript dashboard page. |
| --- | --- |
| GET /data | Returns a JSON object with all current sensor data, maze state, walls, flood values, and log. Polled every 300ms by the browser. |
| GET /command?action=... | Sends a command: FORWARD, BACKWARD, LEFT_90, RIGHT_90, MOVE_DIST, BACK_DIST, TURN_LEFT, TURN_RIGHT, STOP, MAZE_START, MAZE_RESET, RESET_ENC. |
| GET /settings | Updates move speed and turn speed. |
| GET /fusioncfg | Updates sensor fusion weights (wGyro, wEncoder, wLidar). |
| GET /correctcfg | Updates all PD gains (Kp, Kd) and limits. |
| GET /generalcfg | Updates obstacle threshold, wheel size, turn angles, brake settings. |
| GET /mazecfg | Updates maze-specific distances and timing. |

## 6.2 Dashboard Features

- Live maze visualization (Canvas-drawn 8x8 grid with walls, visited cells, path, robot position, and flood values).

- Real-time LiDAR bar charts showing distance on all three sensors.

- IMU readout: yaw, heading lock, gyro Z rate.

- Encoder odometry readout with progress bar.

- Fusion correction visualizer — 4 animated bars showing each layer's correction.

- Robot state badge (e.g., MZ_SCAN, MZ_FWD, STOPPED, etc.).

- Event log — last 30 logged messages with timestamps.

- Tunable PD parameters, maze dimensions, speeds — all live, no recompile needed.

# 7. Summary — How Everything Works Together

When MAZE_START is triggered from the dashboard:

1. The maze data structures are reset and the flood fill is seeded from the center.

2. The robot scans walls at its starting position using 5 LiDAR samples per direction.

3. Flood fill runs and findBestDirection() picks the next cell with the lowest flood value.

4. If a turn is needed: backup → pivot (gyro-tracked) → advance.

5. The robot drives forward one cell (200mm), controlled by the 3-sensor fused PD controller (gyro 55% + encoder 25% + LiDAR 20%).

6. On arrival, it scans walls again and adds any newly discovered walls to the map.

7. If a new wall changes the optimal path, flood fill recalculates all values.

8. Steps 3–7 repeat until the robot reaches one of the four center cells.

9. The robot stops and the state changes to MAZE_DONE.

Throughout the entire process, the web dashboard updates every 300ms, showing the live maze map, sensor readings, corrections, and event log — all without interrupting the robot's control loop.